# METHOD OF STATEMENT

Date: 13th Dec 2017

Description of work

To shift out 5 unit machine and machine parts from production room to loading area, hoist up and load onto wooden base for packing, after packing load onto U-Bed, low loader and deliver to warehouse

Machine Spec

Spray Booth with Sand Blasting

335 x 305 x 145 cm / Wt: 1, 500 kgs

Barmail machine

305 x 180 x 185 cm / Wt: 1, 500 kgs

Sunfirm CST-4680 CNC Lathe

710 x 300 x 278 cm / Wt: 14, 000 kgs

Press machine

200 x 200 x 317 cm

OKK KCV-1000 (Dismantle into parts)

Main Body: 3970 x 3070 x 3490 mm / Wt: 15, 420 kgs

Table: 9520 x 2040 x 2790 mm / Wt: 15, 100 kgs

Risk Assessment

A copy of Risk Assessment will be attach

Date / Time of Operation

18-19/12/17   -   10:00 am

Location of Operation

#### No 8 Tuas West Ave

#### Hunting Energy Services Pte Ltd

Delivery Address

Wyn 2000 Warehouse

No 21 Tuas Ave 9

Equipment Use

1 x 75/100 ton lorry crane

1 x 50 ton lorry crane

1 x forklift

U-Bed/low loader

Rollers/pallet truck

Wire/webbing sling

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and disposed

Lifting Crew

MOM certified lifting supervisor, rigger, signalman and lorry crane operator will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, helmet, etc..)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Jack up machine to 3” high

- Insert rollers to machine base

- Shift out machine from production

- Transport machine to designated hoisting/packing area

- Hoist up machine and load onto wooden base for packing

- After packing, hoist up and load case onto u-bed/low loader

- Deliver machine to customer warehouse

- Unload machine, shift and position aside

- Once job complete, team leader will instruct the team to perform proper housekeeping, all debris will be clear and dispose

- Job complete

Remark:

MOS subject to change by supervisor/lifting supervisor on site

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175