# METHOD OF STATEMENT

Date: 17th Dec 2018

Description of Work

To unload 1 unit CMM machine from lorry chassis, shift and position into customer premise at ground floor

Machine Spec

1 unit Global S Green 07.10.07 CMM

Dim: 1908 x 1340 x 1655 mm

Wt: 2, 026 kgs

Risk Assessment

A copy of Risk Assessment will be attach.

Date / Time of Operation

28/12/2018   –   11:00 am

Location of Operation

#### No 9 Woodlands Ave 9

Republic Polytechnic, School Of Engineering, RPIC 2B/4B

Equipment Use

1 x lorry crane

Pallet truck

Webbing sling

Obstacles

No obstacle on crane parking area and route to machine position

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman, lorry loader operator will be involved in this operation

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Lorry crane to park at designated hoisting area for the lift

- Hoist down machine from lorry chassis and lower down to near ground level

- Swing crane to unloading area

- Insert pallet truck under machine base for the move

- Transport machine into premise at ground floor

- Shift and position machine at designated location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris, if any will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175