# METHOD OF STATEMENT

Date: 05th Nov 2020

Description of work

To hoist down 1 unit machine from 2nd floor, transport to front side of building, hoist up to 3rd floor through wall hacking opening by using platform and mobile/lorry crane

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

12/11/2020

Time of Operation

10:00

Location of Operation

#### Blk 2024 Bukit Batok St 23 #02-42 & #03-32

#### Miller Precision Engineering Pte Ltd

Equipment Use

2 x 633 lorry loader

1 x lorry loader

Pallet truck/rollers

1 x working platform

1 set wire/webbing sling

Shackle

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and mobile/lorry crane operator will be involved in this operation (attached all lifting crew certificate)

P.P.E Required

Safety Shoe

Safety Helmet

Safety Harness

Self Retracting Lifeline

Safety Aspect

Assignment of traffic controller to control movement of vehicle and personnel within the vicinity

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet, safety harness)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deployment of 2 x lorry crane at designated hoisting area

- Setting up of crane at site location

- Hoist up platform to 2nd floor wall hacking opening

- Stabilize platform effectively and ensure platform 3ft metal plate protruding is fully lean and rested on the building opening slab

- Use strong belt to secure front left and right of platform onto building railing/pillar to prevent platform swing outward

- Tied back platform to a competent point and secured using 1 x lorry crane

- 1 x lorry crane to hoist down machine from 2nd floor and load onto lorry chassis

- Transport to front side of building

- Hoist up to 3rd floor through wall hacking opening

- Lower down machine onto platform

- Insert rollers/pallet truck under machine base

- Shift out machine from working platform

- Transport machine into tenant premise

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris, if any will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175