# METHOD OF STATEMENT

Date: 29th May 2023

Description of work

To hoist up 1 unit machine to 3th floor through shutter door opening by using platform and mobile/lorry crane

Machine Spec

1 unit Food Process machine

N/Wt: 2, 500 kg

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

Estimate:

01/06/23

Time of Operation

09:30 am

Location of Operation

#### 22 Chin Bee Rd

Equipment Use

1 x 50 ton mobile crane

2 x lorry loader

1 x platform

Wire sling

Shackle

Rollers/pallet truck

Obstacles

No obstacle on crane parking area and route to equipment positioning

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and mobile/lorry crane operator will be involved in this operation (attached all lifting crew certificate)

P.P.E Required

Safety Shoe

Safety Helmet

Safety Harness

Self Retracting Lifeline

Safety Aspect

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet, safety harness)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M qualified lifting supervisor, rigger and signalman

# Job Scope

- Deployment of mobile/lorry crane at designated hoisting area

- Lorry to transport machine into site premise

- Mobile crane to hoist up platform to 3rd floor shutter gate opening

- Stabilize platform effectively and ensure platform 3ft metal plate protruding is fully lean and rested on the building opening slab

- Tied back platform to competent point and secured using 1 x lorry crane

- Use strong belt to secure left and right side of platform onto building railing/pillar to prevent platform swing outward

- Mobile crane to hoist up machine to 3rd floor

- Lower down machine onto platform

- Insert rollers/pallet truck under machine bed

- Use of manual pulley to control moving speed of machine

- Shift out machine from platform

- Position machine into tenant premise

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani / Kevin Wong

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175