# METHOD OF STATEMENT

Date: 12th Nov 2021

Description of work

To hoist up 2 unit machine to 3rd floor of building through shutter door opening by using platform and mobile/lorry crane

Machine Spec

1 unit Engle LSR 330/120 machine

Wt: 6, 500 kg

1 unit Sodick VL100 machine

Wt: 3, 100 kg

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

19/11/21 (Friday)

Time of Operation

09:45 am

Location of Operation

#### Blk 28 Ayer Rajah Crescent #03-04

Equipment Use

1 x 50ton mobile crane

1 x 633 lorry loader

Pallet truck/rollers

1 x working platform

1 set wire/webbing sling

Shackle

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and mobile/lorry crane operator will be involved in this operation (attached all lifting crew certificate)

Safety Aspect

Assignment of traffic controller to control movement of vehicle and personnel within the vicinity

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet, safety harness)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deployment of mobile/lorry crane at designated hoisting area

- Hoist up platform to 3rd floor shutter door opening

- Stabilize platform effectively and ensure platform 3ft metal plate protruding is fully lean and rested on the building opening slab

- Use strong belt to secure front left and right of platform onto building railing/pillar to prevent platform swing outward

- Tied back platform to a competent point and secured using 1 x lorry crane

- 1 x mobile crane to hoist up machine to 3rd floor

- Lower down machine onto platform

- Insert rollers/pallet truck under machine base

- Shift out machine from working platform

- Position machine at the designated location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175