# METHOD OF STATEMENT

Date: 30th Aug 2023

Description of work

1) To hoist down 1 unit machine from 5th floor of building through swing gate opening by using platform and lorry crane

2) To shift out small machine to loading area via cargo lift

Machine Spec

1 unit Mold Press machine

Dim: 220 x 190 x 270 cm

Wt: 5, 000 kg

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

09/09/23 (Sat)

Time of Operation

09:45 am

Location of Operation

#### Blk 2 Bukit Batok St 24 #05-19

#### Aetec Pte Ltd

Equipment Use

2 x 633 lorry loader

2 x lorry loader

Pallet truck/rollers

1 x working platform

1 set wire sling

Shackle

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry crane operator will be involved in this operation (attached all lifting crew certificate)

P.P.E Required

Safety Shoe

Safety Helmet

Safety Harness

Self Retracting Lifeline

Safety Aspect

Assignment of traffic controller to control movement of vehicle and personnel within the vicinity

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet, safety harness)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Transport lorry to 4th floor via ramp up

- Setting up of crane at site location

- Hoist up platform to 5th floor through swing gate opening

- Stabilize platform effectively and ensure platform 3ft metal plate protruding is fully lean and rested on the building opening slab

- Use strong belt to secure front left and right of platform onto building railing/pillar to prevent platform swing outward

- Tied back platform to a competent point and secured using 1 x lorry crane

- Shift out machine from production into platform

- Insert sling to crane hook and secured onto rigging point of machine

- 1 x lorry crane to hoist down machine to the ground

- Load up machine onto lorry chassis

- Secured machine with strapping belt

- Transport and deliver machine to another location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Kevin Wong / Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175