# METHOD OF STATEMENT

Date: 17th June 2022

Description of work

1) To hoist up 1 unit machine to 3rd floor of building through wall hacking opening by using platform and mobile/lorry crane

2) To hoist down 1 unit Bar Feeder from 3rd floor and load onto lorry chassis

Machine Spec

1 unit Citizen A20 machine

Dim: 1185 x 2120 x 1820 mm

Wt: 2, 200 kg

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

24/06/22

Time of Operation

Afternoon schedule

Location of Operation

#### Blk 2019 Bukit Batok St 23 #03-256

Jit Seng Technical Services

Equipment Use

2 x 633 lorry loader

1 x lorry loader

Pallet truck/rollers

1 x working platform

1 set wire/webbing sling

Shackle

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry crane operator will be involved in this operation (attached all lifting crew certificate)

P.P.E Required

Safety Shoe

Safety Helmet

Safety Harness

Self Retracting Lifeline

Safety Aspect

Assignment of traffic controller to control movement of vehicle and personnel within the vicinity

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet, safety harness)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deployment of lorry crane at designated hoisting area

- Setting up of crane at site location

- Hoist up platform to 3rd floor wall hacking opening

- Stabilize platform effectively and ensure platform 3ft metal plate protruding is fully lean and rested on the building opening slab

- Use strong belt to secure front left and right of platform onto building railing/pillar to prevent platform swing outward

- Tied back platform to a competent point and secured using 1 x lorry crane

- 1 x lorry crane to hoist up machine to 3rd floor

- Lower down machine onto platform

- Insert pallet truck under machine base

- Shift out machine from working platform

- Transport machine into tenant premise

- Position machine at the designated location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani / Kevin

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175