# METHOD OF STATEMENT

Date: 14th June 2022

Description of work

To hoist up 4 unit machine to 2nd floor through swing gate opening by using platform and mobile/lorry crane

Machine Spec

3 unit CV800 CNC

Dim: 2.2m x 2.3m x 2.3m

Wt: 5, 400 kg

1 unit SV1000A CNC

Dim: 3m x 2.3m x 2.4m

Wt: 8, 500 kg

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

02/07/22 (Saturday)

Time of Operation

09:30 am

Location of Operation

#### Blk 3023 Ubi Rd 3 #02-04

#### Sing Geyi Precision Engineering

Equipment Use

1 x 50 ton mobile crane

2-3 x lorry loader

1 x platform

Wire sling

Webbing sling

Shackle

Rollers/pallet truck

Obstacles

No obstacle on crane parking area and route to equipment positioning

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and mobile/lorry crane operator will be involved in this operation (attached all lifting crew certificate)

P.P.E Required

Safety Shoe

Safety Helmet

Safety Harness

Self Retracting Lifeline

Safety Aspect

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet, safety harness)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M qualified lifting supervisor, rigger and signalman

# Job Scope

- Deployment of mobile/lorry crane at designated hoisting area

- Lorry to transport machine into site premise

- Mobile crane to hoist up platform to 2nd floor swing gate opening

- Stabilize platform effectively and ensure platform 3ft metal plate protruding is fully lean and rested on the building opening slab

- Tied back platform to competent point and secured using 1 x lorry crane

- Use strong belt to secure left and right side of platform onto building railing/pillar to prevent platform swing outward

- Mobile crane to hoist up machine to 2nd floor

- Lower down machine onto platform

- Workers to put on safety harness before stepping into the work platform

- Insert rollers/pallet truck under machine bed

- Shift out machine from platform

- Position machine into tenant premise

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani / Kevin Wong

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175