# METHOD OF STATEMENT

Date: 03rd Aug 2020

Description of work

To shift out 1 unit machine and accessories from production room to loading area, hoist and lay down machine, load onto 1 x 40ft FR and 1 x 20ft GP container, secured with lashing belt and choking wood

Machine Spec

1 unit INGYU 220 Press

Dim: 310 x 400 x 160 cm

Wt: 32, 000 kg

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

07-08/09/2020

Time of Operation

10:00 am

Location of Operation

#### 2 Benoi Crescent

SESS

Equipment Use

1 x 70 ton mobile crane

2 x 50 ton lorry crane

1 x forklift

Heavy duty rollers/pallet truck

Wire sling

Shackle

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and disposed

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and mobile/lorry crane operator will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Jack up machine to 4” high

- Insert rollers to machine base

- Shift out machine from production

- Use of forklift to assist in pulling/pushing machine to loading area

- Using 2 x crane to hoist and lay down machine

- Load machine onto container

- Secured machine with lashing belt and choking wood

- Cover machine with canvas

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175