# METHOD OF STATEMENT

Date: 10th May 2021

Description of Work

To unload 10 cases of 1 unit machine from low loader, using gantry crane and chain block to lower down into 4m x 8m hole

Machine Spec

HNKJ-1200 Press machine

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

TBC

Time of Operation

TBC

Location of Operation

Bulim Ave

Shimano (S) Pte Ltd

Equipment Use

1 x 160 ton mobile crane

633 lorry crane

Low loader

1 set of hydraulic gantry crane

10 ton chain block

1 x forklift

Wire sling

Shackle

Obstacles

No obstacles on operation area and route to machine position

Barricade off operation area to prevent person who are not involved enter unintentionally

Environment

No operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Safety Aspect

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Forklift to unload gantry crane from low loader

- To assembly gantry crane inside factory

- Mobile crane to hoist down machine case from low loader to the ground

- Uncrate machine case

- Hoist up and detach machine from base

- Lower down machine to near ground level

- Insert rollers under machine base

- Shift machine into production

- Using gantry crane and 10ton chain block to assist in lowering machine into 8m hole

- Release chain block slowly for machine to sit on the hole

- Repeat process till all machine are rested at the designated location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175