# METHOD OF STATEMENT

Date: 03rd June 2016

Description of Work

To lift up 1 unit lathe machine from 120 cm deep hole, insert rollers, shift out to loading bay, hoist up onto low-loader and deliver to new location

Risk Assessment

A copy of Risk Assessment will be attach

Date / Time of Operation

To be advised

Location of Operation

Equipment Use

1 x mobile crane

1 x lorry crane

1 x low-loader

1 set hydraulic structure

Heavy duty rollers/pallet truck

Wire/webbing sling

1 x forklift

Obstacles

To clear all obstacles on route to machine moving

Environment

No operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, helmet, etc..)

Lifting machine and lifting gear with M.O.M certificate

# JOB SCOPE

- Setting of structure on location

- Fix beam onto structure using customer overhead crane

- Attached wire sling onto beam

- Load machine onto the ground

- Insert rollers/pallet truck to base of stone

- Shift stone from loading area to designated location

- Set up  ‘A’ frame structure at location

- Using chain block attached to ‘A’ frame structure, to lift up stone slightly above ground

- Push structure together with stone to the exact location

- Release chain block slowly and let stone sit on the location inside hole

- Once job complete, team leader will instruct the team to perform proper housekeeping, all debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175