# METHOD OF STATEMENT

Date: 26th Aug 2019

Description of Work

To lift up 1 unit Engine from 1ft cement foundation using ‘A’ frame with chain block, shift out to loading area, hoist up and load onto wooden base for packing, after packing, load onto Malaysia lorry

Risk Assessment

A copy of Risk Assessment will be attach

Date / Time of Operation

28/08/2019   -   09:45 am

Location of Operation

#### 61 Science Park Rd #01-03

Equipment Use

1 x 633 lorry loader

Pallet truck

Wire/webbing sling

1 x ‘A’ frame structure with 3 ton chain block

Wooden case

Obstacles

No obstacles on crane parking area and route to machine position

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry loader operator will be involved in this operation

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# Job scope

- Assemble ‘A’ frame structure and chain block

- Lift up engine from 1ft cement foundation using chain block

- Shift chain block away from foundation

- Lower down engine

- Insert pallet truck under engine bed

- Shift out to loading bay

- Lorry loader to hoist up engine and load onto wooden base

- Pack to form a case

- After packing, load onto Malaysia lorry

- Once job complete, team leader will instruct the team to perform proper housekeeping, all debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175