# METHOD OF STATEMENT

Date: 11th Apr 2024

Description of Work

To Shift out 2 Unit Yamasaki Giken from 42 Lok Yang Way to 40 Lok Yang Way

To Shift 1 unit Wasino A18 Machine from 42 Lok Yang Way (Back) to 42 Lok Yang Way (Front) by hoisting

To shift the following Machine from 40 Lok Yang Way to 42 Lok Yang Way

To Internal Shift 3 Units a-D21 Fanuc Machine within 40 Lok Yang way

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

12/04/24

Time of Operation

09:45 am

Location of Operation

40/42 Lok Yang Way

Equipment Use

1-2 x lorry crane

Rollers/pallet truck

Hydraulic/hand jack

1 x 7 ton forklift

Canvas/metal plate (to lay out for floor protection)

Obstacles

No obstacles on crane parking area

Barricade operation area to prevent person who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman, lorry loader operator and forklift driver will be involved in this operation

Safety Aspect

Assignment of traffic controller to control movement of vehicle and personnel within the vicinity

Barricade operation area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# Job scope

- Deploy Lorry at designated unloading area

- Unload forklift from Lorry

- Using forklift to fork Machine out of premises Transfer to 40 Lok Yang Way

- Shift and position into premises

- Shift Wasino A18 Machine to loading/unloading area for hoisting from the back of the factory drive to front door for unloading.

- Shift and postion machine into final position

- Moving out 2 Fanuc T-21 Using Pallet Trucks from 40 Lok Yang Way transfer to 42 Lok Yang Way

- Internal shifting of 3 units Fanuc D-21 Machine using Pallet Trucks

- Lower down machine to near ground level

- Position machine at the designated location

- Unload all machine from rollers/pallet trucks and insert levelling pad under machine bed

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Kevin Wong / Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175