# METHOD OF STATEMENT

Date: 25th Feb 2025

Description of work

Hoist down steel case from lorry chassis to the ground, uncrate, mount onto metal plate, shift & position into factory premises at ground floor.

Machine Spec

1 steel case of 1 unit Robot Arm

Dim: 290 x 145 x 257 cm / Wt: 1, 300 kg

1 metal base plate

Dim: 150 x 150 x 50mm

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

06/03/25

Time of Operation

11.00am

Location of Operation

1 First Lok Yang Road

Nippon Paint

Equipment Use

1 x lorry loader

1 set webbing sling

Pallet truck

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry crane operator will be involved in this operation (attached all lifting crew certificate)

Safety Aspect

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# Job Scope

- Deploy lorry loader at designated hoisting area

- Hoist down case from lorry chassis to the ground

- Uncrate case and put aside

- Rigger to insert sling to crane hook

- Secured sling to rigging point of robot arm

- Hoist up and mount onto metal plate

- Insert pallet truck under plate bed

- Shift and transport into factory premise at ground floor

- Position at the designated location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Kevin Wong / Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175