# METHOD OF STATEMENT

Date: 19th Fed 2024

Description of work

Hoist down 1 unit LDl Machine from lorry chassis, shift & position into the premises at ground floor.

Machine Spec

1 unit LDl Handler Machne

Dim: 230 x 100 x 200cm/ G Wt: 300kgs

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

22/02/24

Time of Operation

10.00am

Location of Operation

86 Second Lok Yang Road

Singapore Epson Ind. Pte Ltd

Equipment Use

1 x lorry loader

1 set Wire/ webbing sling

Pallet Truck

Metal plate/ canvass for floor protection

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and mobile/lorry crane operator will be involved in this operation (attached all lifting crew certificate)

Safety Aspect

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# Job Scope

- Deploy lorry loader at designated hoisting area

- Insert sling to crane hook and secured onto machine hoisting point

- Lorry loader to hoist up machine from lorry chassis.

- Swing crane and lower down machine to near ground level

- Insert pallet truck under machine bed

- Shift and transport machine into factory premise at ground floor

- Position machine at the designated location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani / Kevin Wong

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175