# METHOD OF STATEMENT

Date:

Description of Work

To hoist down 2 unit machine from 3rd floor by using lorry loader (hoisting via back lane of building)

Machine Spec

2 unit Milling machine

Wt: 1, 600 kg each

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

Time of Operation

10:00 am

Location of Operation

#### Blk

Equipment Use

1 x 633 lorry loader

Pallet truck

Wire/webbing sling

Shackle

Obstacles

No obstacles on crane parking area

Barricade operation area to prevent person who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry loader operator will be involved in this operation

P.P.E Required

Safety shoe

Safety helmet

Safety Aspect

1) Barricade off work area with safety cones, red/white tape and lifting in operation signage

2) All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed

3) Lifting machine and lifting gear with M.O.M certificate

4) Lifting crew shall be composed to the following but not limited by other trained personnel in respect to the number of crew needed for the operation:

a) Lifting Supervisor

b) Lorry Loader Operator

c) Riggerman

d) Signalman

# Job scope

- Deployment of lorry loader at designated hoisting area

- Setting up of lorry loader at site location with outrigger fully extended and resting on steel plate/timber mats as base plate

- Crane operator to extract crane boom to 3rd floor

- Shift out machine to near crane

- Rigger to insert required lifting sling to crane hook

- Secure sling to rigging point of machine

- Lorry loader to hoist up machine 1ft above ground to check on balancing

- When well balanced, hoist up machine above parapet wall

- Crane to slowly swing machine out

- Retract crane boom to lower down machine to the ground

- Load up machine onto lorry chassis

- Secure machine with strapping belt prior to moving out of site premise

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Kevin Wong / Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175