# METHOD OF STATEMENT

Date: 14th Aug 2017

Description of Work

1) To unload 2 unit Lathe machine from lorry chassis, shift and position into the workshop at ground floor

2) To shift out 1 unit used Nantong Milling from workshop and load onto lorry chassis

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

16/08/2017

Location of Operation

#### No 21 Tampines Ave 1

Temasek Polytechnic, Blk 16 #01-68

Equipment Use

1 x lorry crane

Pallet truck

Wire/webbing sling

Obstacles

No obstacles on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

MOM certified lifting supervisor, rigger, signalman, lorry loader operator will be involved in this operation

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, helmet, etc..)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by MOM certified lifting supervisor, rigger and signalman.

# JOB SCOPE

- Lorry crane to park at designated hoisting area for the lift

- Hoist and lower down machine from lorry chassis onto the ground

- Insert pallet truck to machine base for the move

- Shift and position lathe machine into workshop at ground floor

- Shift out 1 unit milling from workshop and load onto lorry

- Once job complete, team leader will instruct the team to perform proper housekeeping, all debris will be clear and dispose

- Job complete

Remark:

MOS subject to change by supervisor/lifting supervisor on site

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175