# METHOD OF STATEMENT

Date: 06th Sep 2018

Description of Work

To unload 3 cases of 1 unit machine from lorry chassis, uncrate, shift and position into customer premise

Machine Spec

TP-32C CNC Wire-Cut

Case 1: 1, 900 kgs

Case 2: 200 kgs

Case 3: 500 kgs

Risk Assessment

A copy of Risk Assessment will be attach

Date / Time of Operation

11/09/2018   -   10:00 am

Location of Operation

#### No 21 Tampines Ave 1

Temasek Polytechnic, School Of Engineering

Equipment Use

1 x lorry crane

Pallet truck

Webbing sling

Obstacles

No obstacles on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman, lorry loader operator will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet, etc..)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman.

# JOB SCOPE

- Lorry crane to park at designated hoisting area for the lift

- Uncrate machine case

- Hoist up and detach machine from base

- Swing crane and lower down machine from lorry chassis to near ground level

- Insert pallet truck to machine base for the move

- Transport machine into premise

- Shift and position machine at the designated location

- Once job complete, team leader will instruct the team to perform proper housekeeping, all debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175