# METHOD OF STATEMENT

Date: 25th Sep 2017

Description of work

1) To hoist up 18 unit Bar Feeder and 10 cases of Material Bar to 6th floor by using platform and mobile crane

2) To hoist/unload 50 unit machine and accessories from loading bay, shift and position to 6th floor via cargo lift

Risk Assessment

A copy of Risk Assessment will be attach

Date Of Operation

04-05/10/2017

Time Of Operation

11:00 am

Location of Operation

#### 10 Tampines North Drive 4 – JTC Space

Turntech Precision Engineering Pte Ltd

Equipment Use

2 x 50 ton mobile crane

Lorry crane

1 x platform

Wire/webbing sling

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent person who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

MOM certified lifting supervisor, rigger, signalman and mobile/lorry crane operator will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

Assignment of traffic controller to control movement of vehicle and personnel within the vicinity

Barricade off work area with safety cones, red/white tape and lifting in operation signage                                                             All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet, safety harness, etc..)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by MOM certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Mobile crane to park at designated hoisting area for the lift

- Hoist up platform to 6th floor opening

- Tied back platform to a competent point and secured using 50 ton mobile crane

- Use strong belt to secured front left and right of platform to prevent platform swing outward

- 50 ton mobile crane to hoist up bar feeder to 6th floor

- Lower down and load bar feeder onto platform

- Insert rollers/pallet truck to bar feeder base

- Shift out bar feeder from platform and  position to the designated location

- Once job complete, team leader will instruct the team to perform proper housekeeping, all debris, if any will be clear and dispose

- Job complete

Remark

MOS subject to change by supervisor/lifting supervisor on site

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175