# METHOD OF STATEMENT

Date: 25th Mar 2019

Description of work

Using mobile crane and platform to hoist down 1 unit machine from 3rd floor to ground floor for packing, after packing load onto 1 x 20ft GP container

Machine Spec

1 unit Shimadzu SMX-600 X-ray machine

Dim: 180 x 170 x 190 cm

Wt: 2, 500 kgs

Risk Assessment

A copy of Risk Assessment will be attach

Date / Time of Operation

28/03/2019   -   Afternoon schedule

Location of Operation

#### 35 Pioneer North Rd

PCI Limited

Equipment Use

1 x 50 ton mobile crane

1 x lorry crane

1 x platform

1 x forklift

Wire/webbing sling

Obstacles

No obstacle on crane parking area.

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and mobile crane operator will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, helmet, safety harness)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Mobile crane to park at designated location for the hoist

- Hoist up platform to 3rd floor opening

- Stabilize platform effectively and ensure it is fully rested on the opening slab

- Use strong belt to secure front left and right of platform onto building railing/pillar to prevent platform swing outward

- Shift out and load machine onto platform

- Strap and secured machine onto platform with strapping belt

- Hoist down platform together with machine to ground floor

- Using forklift to fork out machine from platform and load onto wooden base for packing

- After packing, load case onto container

- Secured case with choking wood

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and disposed

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175