# METHOD OF STATEMENT

Date: 26th Dec 2019

Description of work

To hoist down 1 unit machine from 5th floor to ground floor and put aside

Machine Spec

1 unit X-ray machine

Dim: 130 x 140 x 241 cm

N/Wt: 480 kgs

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

28/12/2019

Time of Operation

09:45 am

Location of Operation

#### 18 Ayer Rajah Crescent

Pepperl + Fuchs Asia PL

Equipment Use

1 x 50 ton mobile crane

1 x lorry crane

1 x platform

1 set wire/webbing sling

Shackle

Obstacles

No obstacle on crane parking area.

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and mobile crane operator will be involved in this operation (attached all lifting crews certificate)

P.P.E Required

Safety shoe

Safety helmet

Safety harness

Self Retracting Lifeline

Safety Aspect

Assignment of traffic controller to control movement of vehicle and personnel within the vicinity

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, helmet, safety harness)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deployment of mobile/lorry crane at designated hoisting area

- 1 x mobile crane to hoist up platform to 5th floor roller shutter opening

- Stabilize platform effectively and ensure platform 3ft metal plate protruding is fully lean and rested on the building opening slab

- Use strong belt to secure front left and right of platform onto building railing/pillar to prevent platform swing outward

- Shift machine and onto platform

- Strap and secured machine onto platform with strapping belt

- Hoist down platform together with machine to ground floor

- Using lorry crane to hoist machine from platform to the ground

- Shift and position machine to temporary location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175