# METHOD OF STATEMENT

Date: 30th Dec 2019

Description of work

To hoist down 1 unit machine from 7th floor through swing gate opening by using platform and mobile crane

Machine Spec

1 unit Emblazing machine

Dim: 350 x 150 x 150 cm

Wt: 3, 500 kgs

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

02/01/2020

Time of Operation

09:45 am

Location of Operation

#### Blk 3016 Bedok North Ave 4 #07-01

Deway Enterprise Pte Ltd

Equipment Use

1 x 50 ton mobile crane

1 x 633 lorry loader

1 x platform

1 set wire/webbing sling

Shackle

Obstacles

No obstacle on crane parking area.

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and mobile/lorry crane operator will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

Assignment of traffic controller to control movement of vehicle and personnel within the vicinity

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet, safety harness)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deployment of mobile/lorry crane at designated hoisting area

- 1 x mobile crane to hoist up platform to 7th floor swing gate opening

- Stabilize platform effectively and ensure platform 3ft metal plate protruding is fully lean and rested on the building opening slab

- Use strong belt to secure front left and right of platform onto building railing/pillar to prevent platform swing outward

- Shift out machine from production and onto platform

- Strap and secured machine onto platform with strapping belt

- Hoist down platform together with machine to ground floor

- Hoist machine from platform and load onto forwarder lorry

- Once job complete, team leader will instruct the team to perform proper housekeeping

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175