# METHOD OF STATEMENT

Date: 27th Dec 2022

Description of work

To hoist down 2 unit cabinets from 3rd floor using platform and lorry loader

Spec

2 unit Cabinets

Dim: 240 x 60 x 226 cm each

Est wt: 1, 000 kg

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

03/01/23

Time of Operation

09:45 am

Location of Operation

#### 14 A.M.K Industrial Pk 2, 3rd floor

Equipment Use

1 x 633 lorry loader

1 x platform

1 set wire/webbing sling

Shackle

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and mobile crane operator will be involved in this operation

P.P.E Required

Safety shoe

Safety helmet

Safety harness

Self Retracting Lifeline

Safety Aspect

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, helmet, safety harness)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deployment of lorry loader at designated hoisting area

- Hoist up platform to 3rd floor roller shutter opening

- Stabilize platform effectively and ensure platform 3ft metal plate protruding is fully lean and rested on the building opening slab

- Use strong belt to secure front left and right of platform onto building railing/pillar to prevent platform swing outward

- Shift cabinets and load onto platform

- Secured cabinet onto platform with strapping belt

- Hoist down platform together with cabinet to the ground

- Load platform onto lorry chassis

- Transport cabinets to another location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175