# METHOD OF STATEMENT

Date: 19th Nov 2018

Description of work

To assist customer engineer in dismantling 1 unit Mitsubishi Vertical Precision Milling machine to smaller parts using factory overhead and lorry crane, load onto low loader/lorry chassis and deliver to new factory

Risk Assessment

A copy of Risk Assessment will be attach

Date / Time of Operation

TBC

Location of Operation

#### 6 Tuas Ave 18

Rohag Singapore Pte Ltd

Equipment Use

1 x 100 ton lorry crane

Factory overhead crane

2 x lorry

1 set wire/webbing sling

1 set shackle

1 x low loader

Rollers/pallet truck

Obstacles

To clear all obstruction in way of crane parking and working area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry crane operator will be involved in this operation

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoe, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# Job Scope

- Transport 100 ton lorry crane into inside factory

- Assist engineer to dismantle and hoist down machine parts

- 1 x low loader to reverse into inside factory

- Load machine onto low loader chassis

- Secured machine with strapping belt prior to moving out of premise and deliver to new factory

- At new factory, unload machine from low loader/lorry chassis

- Shift and position machine at the designated location

- To assist engineer assemble machine parts

- Once job complete, team leader will instruct the team to perform proper housekeeping, all debris will be clear and dispose

Remark:

MOS subject to change by supervisor/lifting supervisor on site

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175