# METHOD OF STATEMENT

Date: 25th June 2019

Description of work

To assist customer engineer to dismantle 1 unit machine into smaller parts using lorry crane, hoist up onto lorry chassis and deliver to scrapyard

Machine Spec

Retrofit Radial Drill 5 Axis CNC Milling – to dismantle into parts

1 x Bed: 400 x 140 x 60 cm / Wt: 4, 500 kgs

1 x Column: 120 x 120 x 330 cm / Wt: 5, 000 kgs

1 x Cross Beam: 300 x 120 x 100 cm / Wt: 3, 000 kgs

1 x Vertical Column: 100 x 100 x 300cm / Wt: 2, 000 kgs

1 x Spindle Head: 400 x 140 x 100 cm / Wt: 2, 000 kgs

1 x Table: 350 x 140 x 170 cm / Wt: 4, 000 kgs

Risk Assessment

A copy of Risk Assessment will be attach

Date / Time of Operation

27-28/06/2019   -   09:30 am

Location of Operation

#### 501 Airport Rd

ST Aerospace Engines Pte Ltd

Equipment Use

1 x 633 lorry loader – PALFINGER PK100002E

1 x 633 lorry loader – PALFINGER PK 92002SHH

1 set wire sling

Shackle

Obstacles

To clear all obstruction in way of crane parking and working area

Barricade operation area to prevent persons who are not involved enter unintentionally

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry crane operator will be involved in this operation

Safety Aspect

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoe, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# Job Scope

- Transport lorry crane into inside factory premise

- Deploy lorry crane at designated hoisting area

- To assist customer engineer to dismantle machine by parts

- Insert sling to machine parts hoisting point

- Using lorry crane to hoist machine parts after dismantle

- Load machine parts onto lorry chassis

- Secured machine parts with strapping belt

- Move out lorry crane from inside factory premise

- Transport and deliver parts to scrapyard

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and disposed

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175