# METHOD OF STATEMENT

Date: 12th Dec 2018

Description of work

To assist customer engineer in dismantling 1 unit Makino EDNC 106 top parts using lorry crane, shift out to loading bay and using forklift to load onto Malaysia low bed

Risk Assessment

A copy of Risk Assessment will be attach

Date / Time of Operation

13/12/2018   -   09:45 am

Location of Operation

#### 5 Tuas Dr 1

Fu Yu Corporation Limited

Equipment Use

1 x 45 ton lorry crane

1 x 10 ton forklift

1 set wire/webbing sling

1 set shackle

1 x Malaysia low bed

Rollers/pallet truck

Obstacles

To clear all obstruction in way of crane parking and working area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman, lorry crane operator will be involved in this operation

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoe, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# Job Scope

- Transport 45ton lorry crane into inside factory

- Shift machine to near crane

- Using lorry crane to assist engineer to dismantle and hoist down machine top part

- Load machine part onto wooden base

- Move lorry crane out from factory

- Shift out machine and part to loading bay

- Using 10 ton forklift to fork up onto Malaysia low bed

- Secured machine with strapping belt prior to moving out of premise and deliver to new location

- Once job complete, team leader will instruct the team to perform proper housekeeping, all debris, if any will be clear and dispose

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175