# METHOD OF STATEMENT

Date: 08th May 2023

Description of work

To assist customer engineer to dismantle machine parts using lorry crane, shift out to loading area, mobile crane to hoist up onto low loader and deliver to new location

Machine Spec

1 unit Wele UB421 Machining Center (dismantle to 3 parts)

Main machine: 10.5 x 3.6 x 3.3 m / Wt: 29, 000 kg

Cross Beam: 3.8 x 2.4 x 3.3 m / Wt: 12, 000 kg

Column:

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

08/05/23 – to shift out accessories

11/05/23 - assist engineer to dismantle machine parts

12/05/23 - to shift out machine from production to loading area, hoist up onto low loader and transport to new location

Location of Operation

#### 25 Pioneer Crescent

#### Voestalpine Specialty Metals Pte Ltd

Equipment Use

2 x lorry loader

1 x 100ton lorry loader

1 x 100ton mobile crane

2 x low loader

Shackle

Wire sling

Heavy duty rollers

Obstacles

To clear all obstruction in way of crane parking and working area

Barricade operation area to prevent persons who are not involved enter unintentionally

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman, lorry and mobile crane operator will be involved in this operation

Safety Aspect

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoe, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# Job Scope

- Transport lorry crane into inside factory premise

- Deploy lorry crane at designated hoisting area

- Insert sling to machine parts rigging point

- To assist customer engineer to dismantle machine by part

- Lorry crane to hoist down machine parts after dismantle

- Load onto lorry chassis

- Shift out to loading area

- Using 100ton mobile crane to hoist up machine and load onto low bed

- Secured machine with belts

- Transport and deliver machine to another location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and disposed

- Job complete

Best Regards

Kevin Wong / Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175