# METHOD OF STATEMENT

Date: 12th Feb 2025

Description of work

To assist customer engineer to dismantle machine parts using lorry crane, shift out to loading area for packing

Machine Spec

UCHINE machine

Refer to attached

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

Estimate:

24/02/25  -  04/03/25

Location of Operation

#### 51 Corporation Rd

#### Avitron Pte Ltd

Equipment Use

1 x lorry loader

Shackle

Wire sling

Heavy duty rollers

Obstacles

To clear all obstruction in way of crane parking and working area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose after job completion

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry crane operator will be involved in this operation

Safety Aspect

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoe, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# Job Scope

- Transport lorry crane into inside factory premise

- Deploy lorry crane at designated hoisting area

- Rigger to insert sling to crane hook

- Secure sling to rigging point of machine parts

- To assist customer engineer to dismantle machine by part

- Lorry crane to hoist down machine parts after dismantle

- Load onto lorry chassis

- Shift out to loading area

- Hoist down and load onto wooden base for packing

- After packing, load up onto customer arrange transport

- Secured case with lashing belts and chocking wood

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and disposed

- Job complete

Best Regards

Kevin Wong / Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175