# METHOD OF STATEMENT

Date: 31st Oct 2018

Description of work

To assist customer engineer in dismantling machine to smaller parts using overhead crane, shift out from production to loading bay, hoist up and load onto 1 x 40ft FR and 1 x 40ft HQ container

Machine Spec

1 unit Daewoo DB 250T Horizontal machine – to dismantle into parts

ATC: 300 x 140 x 120 cm / Wt: 2 ton

Chip Conveyor: 500 x 115 x 160 cm / Wt: 1 ton

Column: 320 x 170 x 120 cm / Wt: 5 ton

Spindle Head: 300 x 100 x 100 cm / Wt: 2 ton

Table: 400 x 220 x 130 cm / Wt: 7 ton

Control Panel: 180 x 70 x 200 cm / Wt: 1 ton

Bed: 500 x 300 x 55 cm / Wt: 12 ton

Est Total Wt: 30, 000 kgs

Risk Assessment

A copy of Risk Assessment will be attach

Date / Time of Operation

02/11/2018   -    10:00 am

Location of Operation

#### 4 Tuas Ave 13

IEG Oilfield Services Pte Ltd

Equipment Use

1 x 50 ton mobile crane

1 x 50 ton lorry crane

Wire/webbing sling

Shackle

Obstacles

To clear all obstruction in way of crane parking and working area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and mobile/lorry crane operator will be involved in this operation

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoe, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# Job Scope

- Transport of lorry crane into production area

- Assist engineer to dismantle machine parts

- Hoist down machine part and put aside

- On another day, load machine and parts onto wooden base for packing

- After packing, deliver to customer warehouse

- Once job complete, team leader will instruct the team to perform proper housekeeping, all debris, if any will be clear and dispose

Remark:

MOS subject to change by supervisor/lifting supervisor on site

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175