# METHOD OF STATEMENT

Date: 25th Feb 2019

Description of work

To assist customer engineer in dismantling 1 used Hartford Double Column machine using factory overhead, after dismantle load onto Malaysia lowbed

Risk Assessment

A copy of Risk Assessment will be attach

Date / Time of Operation

28/02/2019   -   09:45 am

Location of Operation

#### 3 Joo Koon Circle

AME International Pte Ltd

Equipment Use

1 x 30 ton factory overhead crane

1 set wire sling

1 set shackle

Malaysia low loader

Obstacles

To clear all obstruction in way of crane parking and working area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger and signalman will be involved in this operation

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoe, safety helmet)

Lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# Job Scope

- To assist engineer to dismantle and hoist down machine part using customer overhead crane

- After dismantle, to reverse Malaysia lowbed into inside factory

- Load machine onto lowbed chassis

- To provide 2 pcs I beam to support cross beam during transportation

- Secured machine with strapping belt

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris, if any will be clear and dispose

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175