# METHOD OF STATEMENT

Date: 25th Feb 2019

Description of work

To assist customer engineer in dismantling 1 unit machine using factory overhead, after dismantle load onto low-loader/lorry chassis, transport to new location, unload and assemble using factory overhead crane

Risk Assessment

A copy of Risk Assessment will be attach

Date / Time of Operation

28/02/2019   -   09:45 am

Location of Operation

#### Pte Ltd

Equipment Use

2 x lorry loader

1 x low-loader

1 x 40ton factory overhead crane (30 Tuas South Ave 8)

1 x 20ton factory overhead crane (2 Gul Circle)

Wire sling (4ton, 8ton)

Shackle (8.5ton, 12ton, 17ton)

Obstacles

To clear all obstruction in way of crane parking and working area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger and signalman will be involved in this operation

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoe, safety helmet)

Lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# Job Scope

- To assist engineer to dismantle and hoist down machine part using customer overhead crane

- After dismantle, to reverse Malaysia lowbed into inside factory

- Load machine onto lowbed chassis

- To provide 2 pcs I beam to support cross beam during transportation

- Secured machine with strapping belt

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris, if any will be clear and dispose

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175