# METHOD OF STATEMENT

Date: 20th Aug 2020

Description of work

To assist customer engineer in dismantling machine to smaller parts using factory overhead crane, after dismantle to hoist up onto low loader/lorry chassis and deliver to new location,

Machine Spec

1 unit G & L VTC 2000 – to dismantle into parts

Column: 280 x 130 x 370 cm / Wt: 14, 000 kg

Oil Tank: 200 x 100 x 180 cm

Chiller: 140 x 140 x 180 cm

Control: 200 x 100 x 220 cm

Base: 500 x 280 x 100 cm / Wt: 18, 800 kg

Cross Beam: 500 x 120 x 100 cm / Wt: 3, 000 kg

Spindle Head: 450 x 100 x 100 cm / Wt: 2, 000 kg

Chip Conveyor: 600 x 80 x 280 cm

ATC: 200 x 200 x 200 cm

Scissor working platform: 300 x 240 x 100 cm

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

TBA

Time of Operation

TBA

Location of Operation

#### 30 Tuas South Ave 8

Cameroon (S) Pte Ltd

Equipment Use

2 x 633 lorry loader

2 x low-loader

Wire sling (4ton, 8ton)

Shackle (8.5ton, 12ton, 17ton)

1 x 40ton factory overhead crane

Obstacles

To clear all obstruction in way of working area

Barricade operation area to prevent persons who are not involved enter unintentionally

Lifting Crew

M.O.M certified lifting supervisor, rigger and signalman will be involved in this operation

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoe, safety helmet, safety harness)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# Job Scope

- To assist engineer to dismantle machine parts

- Riggerman to secured sling onto lifting point of load to be lifted

- Using factory overhead crane to lift up

- Hoist down machine part and put aside

- On another day, shift out machine and parts from production to loading area

- Hoist up and load onto low loader/lorry chassis

- Secured machine with strapping belt prior to moving out of premise and deliver to new factory

- At new factory, unload machine from low loader/lorry chassis

- Shift and position machine at the designated location

- To assist engineer assemble machine parts using factory overhead crane

- Once job complete, team leader will instruct the team to perform proper housekeeping, all debris will be clear and dispose

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175