# METHOD OF STATEMENT

Date: 04th July 2025

Description of work

To hoist up container from the ground onto container chassis

Spec

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

17/07/25

Time of Operation

09:30 am

Location of Operation

#### No 11 Tuas South Ave 12

#### Halliburton Completions MFG Pte Ltd

Equipment Use

1 x 633 lorry loader

Wire sling

Shackle

Obstacles

No obstacle on loading and crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and disposed

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry crane operator will be involved in this operation

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

Lifting machine and lifting gears with M.O.M certified

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy lorry loader at designated loading area

- Set up crane with outriggers fully extended and resting on steel plate/timber mats as base plate

- Rigger to insert required sling to crane hook

- Secure sling to rigging point container

- Hoist up container from the ground and load up onto container chassis

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Kevin Wong / Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175