# METHOD OF STATEMENT

Date: 13th Oct 2023

Description of work

To shift out machines from tenant premise, using lorry loader to hoist and lay down machine, load up onto Malaysia lorry

Machine Spec

1 unit Aida NC1-60 Press / Wt: 3, 000 kg

1 unit Aida NC1-110 Press / Wt: 11, 00 kg

1 unit Aida NC1-150 Press / Wt: 15, 000 kg

1 unit Straightener

2 x pallet

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

25/10/23

Time of Operation

09:45 am

Location of Operation

#### 25B Loyang Crescent, Blk 302 #01-08

Aureumaex Industries (S) Pte Ltd

Equipment Use

1 x 55ton lorry loader

1 x 633 lorry loader

2 x Malaysia lorry

Heavy duty rollers

Wire sling/shackle

Obstacles

No obstacle on crane parking area

Barricade off operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and disposed

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry loader operator will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet, safety harness)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by MOM certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy lorry loader at designated hoisting area

- Shift out machine to loading near crane

- Attached required lifting gears to crane hook and secured to rigging point of machine

- 1 x lorry loader to hoist up machine

- 1 x lorry loader for assist to lay down machine to horizontal position

- Hoist up machine and load onto Malaysia lorry

- Release all sling from machine rigging point

- Secured machine with strapping belt/chain prior to being transport out from tenant premise

- Transport and deliver machine new location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Kevin Wong / Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175