# METHOD OF STATEMENT

Date: 12th Apr 2021

Description of work

To hoist down 1 unit machine from lorry chassis, load onto platform, shift and position into customer premise at ground

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

Est: 17-24/04/21 (Saturday)

Time of Operation

10:00 am

Location of Operation

#### Blk 115A Commonwealth Drive #01-05

Acot Precision Engineering Pte Ltd

Equipment Use

2 x 633 lorry loader

1 x platform/metal plate - to lay out for machine pathway

Heavy duty rollers/pallet truck

1 set wire sling

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry crane operator will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

Assignment of traffic controller to control and guide movement of vehicle and personnel within the vicinity

Barricade off work area with safety cones/red white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy lorry crane at designated location for the lifting

- Lay out platform directly in front of tenant door entrance for machine pathway

- Hoist down machine from lorry chassis

- Load machine onto platform

- Insert rollers/pallet truck under machine base

- Shift out machine from platform into tenant premise at ground floor

- Position machine at the designated location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

REMARK:

To dismantle wooden door, door frame and make good upon job completion

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175