# METHOD OF STATEMENT

Date: 02nd Nov 2020

Description of work

To unload 4 cases of 1 unit machine from lorry chassis to the ground, uncrate, hoist up and load onto platform, shift and position into customer premise at ground

Machine Spec

NXV 1680A CNC machine

Case 1: 4.49m x 2.31m x 2.55m / Wt: 9, 000 kg

Case 2: 3.66m x 2.19m x 2.32m / Wt: 330 kg

Case 3: 2.94m x 2.12m x 1.30m / Wt: 760 kg

Case 4: 5.76m x 0.67m x 2.00m / Wt: 620 kg

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

07/11/2020 (Saturday)

Time of Operation

13:00 pm

Duration of Operation

13:00pm – 18:00pm

Location of Operation

#### Blk 5012 A.M.K Ave 5 #01-04

Akribis System Pte Ltd

Equipment Use

3 x 633 lorry loader

1 x platform - to lay out for machine pathway

Rollers/pallet truck

1 set wire sling

Shackle

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry crane operator will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

Assignment of traffic controller to control and guide movement of vehicle and personnel within the vicinity

Barricade off work area with safety cones/red white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy lorry crane at designated location for the lifting

- Hoist down and lay out platform directly in front of tenant door entrance for machine pathway

- Hoist down case from lorry chassis to the ground

- Uncrate machine case and put aside

- Hoist up and detach machine from base

- Load machine onto platform

- Insert rollers under machine base

- Shift out machine from platform into tenant premise at ground floor

- Position machine at the designated location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

REMARK:

To dismantle sun breaker at tenant premise and make good upon job completion

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175