# METHOD OF STATEMENT

Date: 15th Jan 2020

Description of work

To uncrate case, using forklift to detach machine from base, load onto platform, shift and position into customer premise at ground

Machine Spec

1 unit Laser machine

Case Dim: 275 x 256 x 226 cm

Wt: 8, 000 kgs

Risk Assessment

A copy of Risk Assessment will be attach

Date / Time of Operation

18/01/2020   -   After 1 pm

Duration of Operation

1pm – 5pm

Location of Operation

#### Blk 5012 A.M.K Ave 5 #01-05

Akribis Systems Pte Ltd

Equipment Use

1 x 633 lorry loader

1 x low loader – to transport forklift

1 x 10 ton forklift

1 x platform - to lay out for machine pathway

Heavy duty rollers/pallet truck

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Safety Aspect

Assignment of traffic controller to control and guide movement of vehicle and personnel within the vicinity

Barricade off work area with safety cones/red white tape and signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

M.O.M certified forklift operator

# JOB SCOPE

- Lay out platform directly in front of tenant door entrance for machine pathway

- Uncrate machine case

- Fork up and detach machine from base

- Load machine onto platform

- Insert rollers under machine base

- Shift out machine from platform into tenant premise at ground floor

- Position machine at the designated location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175