# METHOD OF STATEMENT

Date: 13th Feb 2020

Description of work

To unload 2 cases of 1 unit machine from low loader/lorry chassis, uncrate, hoist up and load onto platform, shift and position into customer premise at ground

Machine Spec

2 cases of 1 unit TTC10 machine

Case 1: 4000 x 2650 x 2530 mm

Wt: 9, 000 kgs

Case 2: 4400 x 1800 x 1700 mm

Wt: 1, 800 kg

Risk Assessment

A copy of Risk Assessment will be attach

Date / Time of Operation

26/02/2020

Duration of Operation

Location of Operation

#### Blk 5012 A.M.K Ave 5 #01-10

LHY Engineering (S) Pte Ltd

Equipment Use

2 x 633 lorry loader

1 x low loader

1 x platform - to lay out for machine pathway

Heavy duty rollers/pallet truck

1 set wire sling/shackle

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry crane operator will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

Assignment of traffic controller to control and guide movement of vehicle and personnel within the vicinity

Barricade off work area with safety cones/red white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy lorry crane at designated location for the lifting

- Lay out platform directly in front of tenant door entrance for machine pathway

- Hoist down case from low loader to the ground

- Uncrate machine case and put aside

- Hoist up and detach machine from base

- Load machine onto platform

- Insert rollers under machine base

- Shift out machine from platform into tenant premise at ground floor

- Position machine at the designated location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175