# METHOD OF STATEMENT

Date: 02nd July 2025

Description of work

To unload 1 unit machine from lorry chassis and load onto platform, shift and position into customer premise at ground floor

Machine Spec

1 unit TTC-10 CNC

Dim: 300 x 240 x 195 cm

Wt: 10, 000 kg

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

Time of Operation

Duration of Operation

09:45 am  –  14:00 pm

Location of Operation

#### Blk 5012 A.M.K Ave 5 #01-10

LHY Engineering (S) Pte Ltd

Equipment Use

2 x 633 lorry loader

1 x platform - to lay out for machine pathway

Rollers/pallet truck

1 set wire sling

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry crane operator will be involved in this operation (attached all lifting crew certificate)

Safety Aspect

Assignment of traffic controller to control and guide movement of vehicle and personnel within the vicinity

Barricade off work area with safety cones/red white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy lorry loader at designated location for the lifting

- Set up crane with outriggers fully extended and resting on timber mats as base plate

- Hoist down platform and lay out directly in front of tenant door entrance for machine pathway

- Hoist down machine from lorry chassis

- Load machine onto platform

- Insert rollers under machine bed

- Shift out machine from platform

- Transport machine into tenant premise at ground floor

- Position machine at the designated location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Kevin Wong / Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175