# METHOD OF STATEMENT

Date: 09th July 2025

Description of work

To hoist down 1 unit machine from lorry chassis to the ground, shift and position into production

Machine Spec

1 unit Tigo CMM

Dim: 130 x 110 x 170 cm

Wt: 800 kg

Date of Operation

10/07/25

Time of Operation

11:30 am

Location of Operation

21 Changi North Crescent

Windsor Airmotive Asia Pte Ltd

Equipment Use

1 x lorry loader

Webbing sling

Pallet truck

Crowbar

Woodblock

Obstacles

Ensure no obstacles at unloading area

Barricade operation area to prevent person who are not involved enter unintentionally

Environment

No operation will be carried out on heavy rain, thunderstorms and lightning weather

All debris will be clear and disposed

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman, lorry crane operator will be involved in this operation (attached all lifting crew certificate)

Safety Aspect

All personnel at site to ensure that risk control are known and supervisor has to ensure that risk control are in place

All our workers are fully equipped with standard PPE (safety shoe, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# Job Scope

# Deploy lorry crane at designated unloading area

# Rigger to insert sling to crane hook

# Secured sling to rigging point machine

# Hoist up machine from lorry chassis

# Swing crane and lower down machine to near ground level

# Insert pallet truck under machine bed

# Transport into production area

# Position at the designated location

# Once job complete, team leader will instruct the team to perform proper housekeeping

# All debris will be clear and disposed

# Best Regards

# Kevin Wong / Zailani

# Eric Wong Machinery Transportation Pte Ltd

# Tel: 62824175