# METHOD OF STATEMENT

Date: 24th Jan 2024

Description of work

To hoist down 1 case from lorry chassis to the ground, uncrate, shift and position machine into factory premise QA room at ground floor

Machine Spec

Global S 15.22.10 CMM

Case Dim: 3928 x 2478 x 2709 mm

Case Wt: 8, 200 kg

Date of Operation

29/01/24 (Mon)

Time of Operation

09:40 am

Location of Operation

1 Seletar Aerospace Heights

Wah Son Engineering Pte Ltd

Equipment Use

1-2 x 633 lorry loader

Rollers

Pallet truck

Wire/webbing sling

Shackle

Hydraulic/hand jack

Woodblock

Canvas/metal plate (to lay out for floor protection)

Obstacles

Ensure no obstacles at unloading area

Barricade operation area to prevent person who are not involved enter unintentionally

Environment

No operation will be carried out on heavy rain, thunderstorms and lightning weather

All debris will be clear and disposed

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry crane operator will be involved in this operation (attached all lifting crew certificate)

Safety Aspect

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel at site to ensure that risk control are known and supervisor has to ensure that risk control are in place

All our workers are fully equip with standard PPE (safety shoe, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# Job Scope

# Deploy lorry crane at designated unloading area

# Insert sling under case

# Lorry crane to hoist up case and unload to the ground

# Uncrate case and put aside

# Rigger will attached sling to crane hook

# Secured sling to rigging point of machine

# Hoist and detach machine from base

# Swing machine away from base

# Lower down machine to near ground level

# Insert rollers under machine bed

# Shift and transport machine into factory premise

# Position at the designated QA room

# Once job complete, team leader will instruct the team to perform proper housekeeping

# All debris will be clear and disposed

# Job complete

# Best Regards

# Kevin Wong / Zailani

# Eric Wong Machinery Transportation Pte Ltd

# Tel: 62824175