# METHOD OF STATEMENT

Date: 12th Jan 2024

Description of work

To unload 1 case from lorry chassis to the ground, shift and position machine into factory premise without uncrating

Machine Spec

1 case Global 07.10.07 CMM

Case Dim: 3318 x 1818 x 1959 mm

Wt: 1, 786 kg

Date of Operation

15/01/24

Time of Operation

09:40 am

Location of Operation

38 Changi North Crescent

Windsor Airmotive Asia Pte Ltd

Equipment Use

1 x 633 lorry loader

Pallet truck

Wire/webbing sling

Shackle

Obstacles

Ensure no obstacles at unloading area

Barricade operation area to prevent person who are not involved enter unintentionally

Environment

No operation will be carried out on heavy rain, thunderstorms and lightning weather

All debris will be clear and disposed

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry crane operator will be involved in this operation (attached all lifting crew certificate)

Safety Aspect

All personnel at site to ensure that risk control are known and supervisor has to ensure that risk control are in place

All our workers are fully equip with standard PPE (safety shoe, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# Job Scope

# Deploy lorry crane at designated unloading area

# Insert sling under case

# Lorry crane to hoist up case

# Swing crane and lower down case to the ground

# Insert pallet truck under case bed

# Shift and transport case to temporary storage area

# Position at the designated location

# Once job complete, team leader will instruct the team to perform proper housekeeping

# All debris will be clear and disposed

# Job complete

# Best Regards

# Kevin Wong / Zailani

# Eric Wong Machinery Transportation Pte Ltd

# Tel: 62824175