# METHOD OF STATEMENT

Date: 26th Dec 2023

Description of work

To unload 1 case from lorry chassis to the ground, uncrate, shift and position machine into factory premise

Machine Spec

1 unit SF 7107 CMM

Case Dim: 279 x 182 x 233 cm

Wt: 2, 177 kg

Date of Operation

Estimate:

02/01/24

Time of Operation

09:40 am

Location of Operation

38 Changi North Crescent

Windsor Airmotive Asia Pte Ltd

Equipment Use

1-2 x lorry loader

Pallet truck

Wire/webbing sling

Obstacles

Ensure no obstacles at unloading area

Barricade operation area to prevent person who are not involved enter unintentionally

Environment

No operation will be carried out on heavy rain, thunderstorms and lightning weather

All debris will be clear and disposed

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry crane operator will be involved in this operation (attached all lifting crew certificate)

Safety Aspect

All personnel at site to ensure that risk control are known and supervisor has to ensure that risk control are in place

All our workers are fully equip with standard PPE (safety shoe, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# Job Scope

# Deploy lorry crane at designated unloading area

# Insert sling under case

# Lorry crane to hoist up case and unload to the ground

# Uncrate case and put aside

# Rigger to attached sling to crane hook

# Secured sling to rigging point of machine

# Hoist and detach machine from base

# Swing machine away from base

# Lower down machine onto machine stand

# Insert pallet truck onto stand

# Shift and transport machine into factory premise

# Position at the designated location

# Once job complete, team leader will instruct the team to perform proper housekeeping

# All debris will be clear and disposed

# Job complete

# Best Regards

# Zailani

# Eric Wong Machinery Transportation Pte Ltd

# Tel: 62824175