# METHOD OF STATEMENT

Date: 17th July 2024

Description of work

To hoist up 1 case onto lorry chassis, deliver to another factory, unload, uncrate, shift and position to 3rd floor via cargo lift

Machine Spec

1 case Global 07.10.07 CMM

Case Dim: 3318 x 1818 x 1959 mm

Wt: 2, 026 kg

Date of Operation

31/07/24 (Wed)

Time of Operation

10:00 am

Location of Operation

Windsor Airmotive Asia Pte Ltd

Move out: 38 Changi North Crescent

Move in: 65 Changi North Crescent

Equipment Use

1 x 633 lorry loader

Pallet truck

Wire/webbing sling

Shackle

Obstacles

Ensure no obstacles at unloading area

Barricade operation area to prevent person who are not involved enter unintentionally

Environment

No operation will be carried out on heavy rain, thunderstorms and lightning weather

All debris will be clear and disposed

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry crane operator will be involved in this operation (attached all lifting crew certificate)

Safety Aspect

All personnel at site to ensure that risk control are known and supervisor has to ensure that risk control are in place

All our workers are fully equip with standard PPE (safety shoe, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# Job Scope

# Deploy lorry crane at designated unloading area

# Insert sling under case

# Lorry crane to hoist and load up case onto lorry chassis

# Secured case with strapping belt

# Transport to another factory

# Hoist down case to the ground

# Uncrate case and put aside

# Insert sling to crane hook

# Secured sling to rigging point of machine

# Hoist up machine from base and load onto machine stand

# Using pallet truck to transport to 3rd floor via cargo lift

# Position at the designated location

# Once job complete, team leader will instruct the team to perform proper housekeeping

# All debris will be clear and disposed

# Job complete

# Best Regards

# Kevin Wong / Zailani

# Eric Wong Machinery Transportation Pte Ltd

# Tel: 62824175