# METHOD OF STATEMENT

Date: 09th Dec 2024

Description of work

To hoist down 1 unit machine from lorry chassis to the ground, shift and position into factory premise at ground floor

Machine Spec

Microweily TY2680

Dim: 369 x 122 x 160 cm

Wt: 2, 850 kg

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

20/12/24

Time of Operation

09:45 am

Location of Operation

77 Joo Koon Circle

Hoebiger

Equipment Use

1 x 633 lorry loader

Rollers/pallet truck

Wire/webbing sling

Crowbar

Woodblock

Obstacles

Ensure no obstacles at unloading area

Barricade operation area to prevent person who are not involved enter unintentionally

Environment

No operation will be carried out on heavy rain, thunderstorms and lightning weather

All debris will be clear and disposed

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry crane operator will be involved in this operation (attached all lifting crew certificate)

Safety Aspect

All personnel at site to ensure that risk control are known and supervisor has to ensure that risk control are in place

All our workers are fully equip with standard PPE (safety shoe, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# Job Scope

# Deploy lorry crane at designated unloading area

# Rigger to attached sling to crane hook

# Secured sling to rigging point of machine

# Hoist and detach machine from base

# Swing machine away from base

# Lower down machine to near ground level

# Insert rollers/pallet truck under machine bed

# Shift and transport machine into factory premise

# Position at the designated location

# Unload machine from rollers and insert levelling pad under machine bed

# Once job complete, team leader will instruct the team to perform proper housekeeping

# All debris will be clear and disposed

# Job complete

# Best Regards

# Kevin Wong / Zailani

# Eric Wong Machinery Transportation Pte Ltd

# Tel: 62824175