# METHOD OF STATEMENT

Date: 15th Dec 2021

Description of work

To hoist down equipment from ground floor to 2nd floor through shutter door opening by using lorry crane

Equipment Spec

Refer to quotation

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

15/12/21 – 15/02/22

Time of Operation

As per required schedule

Location of Operation

#### 1 Changi North St 2

ETLA Limited

Equipment Use

633 FALFINGER lorry loader

Wire/webbing sling

Shackle

Rollers/pallet ruck

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose.

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry crane operator will be involved in this operation

Safety Aspect

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet, safety harness) Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy lorry loader at designated hoisting area

- Shift equipment to near shutter door opening

- Insert sling to crane hook and secured onto equipment hoisting point

- Hoist down equipment from 2nd floor to the ground

- Load equipment onto lorry chassis or put aside

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175