# METHOD OF STATEMENT

Date: 28th June 2022

Description of work

To hoist up 1 unit welding table to 3rd floor through shutter door opening by using lorry crane

Equipment Spec

Welding Table

Dim: 320 x 170 x 40 cm / Wt: 1, 515 kg

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

29/06/22

Time of Operation

Afternoon schedule

Location of Operation

#### 2 Changi North St 2

ETLA Limited

Equipment Use

633 FALFINGER lorry loader

Wire/webbing sling

Shackle

Rollers/pallet ruck

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose.

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry crane operator will be involved in this operation

Safety Aspect

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet, safety harness) Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy lorry loader at designated hoisting area

- Shift out table to near crane

- Insert sling to crane hook and secured onto welding table rigging point

- Hoist up table to 3rd floor through shutter door opening

- Lower down table to the ground

- Insert pallet truck to table bed

- Shift and position to designated location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175