# METHOD OF STATEMENT

Date: 14th Apr 2025

Description of work

Transport lorry crane into inside production area, hoist up machine column and reposition

Spec

Machine column

Wt: 5, 000 kg

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

21/04/25

Time of Operation

09:30 am

Location of Operation

149 Gul Circle

FMC Technologies Singapore Pte Ltd

Equipment Use

1 x 633 lorry crane

Wire sling

Webbing sling

Shackle

Obstacles

Clear obstruction in way of working area and route to structure position

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry crane operator will be involved in this operation

Safety Aspect

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoe, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# Job Scope

- Transport lorry crane into inside production area

- Deploy lorry loader at designated hoisting location

- Set up crane with outrigger extended and resting on timber mats as base plate

- Riggers to insert sling to crane hook

- Secured sling to rigging point of machine column

- Hoist up machine column and reposition to required location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris or wood waste will be clear and dispose if necessary

- Job complete

Best Regards

Kevin Wong / Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175