# METHOD OF STATEMENT

Date: 10th Aug 2021

Description of work

To unload 3 unit machine with accessories from customer arrange container to the ground, uncrate, load onto lorry chassis, transport lorry into production area, using customer overhead crane to unload and position to designated location

Machine Spec

2 unit Radial Boring machine

Dim: 4.10 x 1.20 x 1.57 m

Wt: 3, 000 kg

1 unit Dahlih MCV-1020A

Dim: 3.40 x 2.60 x 2.77 m

Wt: 7, 050 kg

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

17-20/08/21

Time of Operation

09:45 am

Location of Operation

#### 22 Jalan Terusan

NOV

Equipment Use

2 x 633 lorry loader

12.5ton factory overhead crane

Wire sling

Shackle

Obstacles

Clear obstruction in way of working area and route to machine position

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry loader operator will be involved in this operation

Safety Aspect

Barricade off work area safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoe, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by MO.M certified lifting supervisor, rigger and signalman

# Job Scope

- Deploy lorry loader at designated location (back to back with container)

- Pull out machine case from container and load onto lorry chassis

- Hoist down case to the ground

- Uncrate case

- Hoist up and detach machine from base

- Load machine onto lorry chassis

- Transport lorry into factory production area

- Using customer 12.5ton overhead crane to unload machine from lorry chassis

- Shift and position to designated location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175