# METHOD OF STATEMENT

Date: 21st Mar 2024

Description of work

Transport lorry crane into inside customer premise, hoist up machine part to assemble

Spec

Machine part

Wt: 10, 000 kg

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

26/03/24

Time of Operation

10:00 am

Location of Operation

1 Seletar Aerospace Crescent

Rolls-Royce Singapore Pte Ltd

Equipment Use

1 x 633 lorry crane

Wire sling

Shackle

Canvas/metal plate (to lay out for floor protection)

Obstacles

Clear obstruction in way of working area and route to machine position

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

Use of extension hose and fix onto lorry exhaust pipe to prevent smoke entering into factory premise

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry crane operator will be involved in this operation

Safety Aspect

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoe, safety helmet)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# Job Scope

- Transport lorry crane into inside customer factory

- Deploy lorry loader at designated hoisting location

- Riggers to insert sling to crane hook

- Rotate crane towards machine part

- Secured sling to rigging point of part

- Hoist up part

- Swing crane to actual location of machine part

- Mazak engineer to assemble part

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Kevin Wong / Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175